Name:__________________

Date:___________________

Period:__________________

Reading to Perform A Task:

Swing Thing

When you swing back and forth on a playground swing, you're modeling the motion of a simple pendulum.  What affects the way a pendulum swings?  Its length?  How far it is pulled back before you release it?  The weight of the object hanging from it?  When the important characteristics of pendulum are identified, you can use math to make predictions about its motion.

Objectives

In this activity you will:

- Construct a pendulum.

- Create a motion plot for a swinging pendulum.

- Identify characteristics that affect a pendulum's motion.

Materials

- CBR unit

- TI graphing calculator (73, 82, 83)

- Calculator to CBR cable

- Meter stick

- String

- Scissors

- Empty aluminum can with pull-tab

- Funnel

- Sand

Procedures to Set up CBR

- Connect the CBR to the calculator using the link cable.

- Turn on your calculator.  If you have not already loaded RANGER program into your calculator, follow these steps.

  - Press 2nd, link, right arrow, enter.  The calculator displays Waiting…

  - Press the 82/83 transfer button on the CBR.

- Run the RANGER program on your calculator.

  - Press prgm.

  - Choose RANGER.

  - Press enter.

- From the MAIN MENU, select 2:  SET DEFAULTS.

5.  With the right arrow at START NOW, press enter.

Procedures to Collect the Data

1.  Tie one end of the string to an empty aluminum can.  Measure 50 centimeters of the

string and cut the string at this point.

2.  Place the CBR on a table or desk.  Position the pendulum about 1 meter away from it.

Be sure you are holding the string by its end.

3.  Aim the sensor at the can.  Pull the can back about 10 centimeters and release it so that

it swings toward and away from the sensor.  When you are ready to begin collecting

data, press enter.

4.  Your plot should show a smooth, wave-like

plot.  If you are satisfied with your plot,

sketch it to the right and go on to the next

section.  If not, press enter, and select 3:

REPEAT SAMPLE from the PLOT

MENU, and try again.

Analyzing the Results

1.  A pendulum completes a cycle as it moves from one extreme position to the other and

back again.  How many complete cycles are displayed on the calculator's screen?

_____________________________________________________________________

2.  The time required for the pendulum to complete a cycle is called the period.  To find

your pendulum's period, use the right and left arrow to trace to the first peak and

record the time (x-value) below.

First Peak:___________________seconds

Trace to the second peak and record the (x-value) below.

Second Peak:___________________seconds

When you have finished tracing the data, press enter.  From the PLOT MENU, select

5:  QUIT.  Press clear.

3.  The period is the difference between the two time peaks.  Find the period and record it

below.

Second Peak:

-     First Peak:

Period:                        seconds

4.  How do you think the period would be affected if a heavier can were used?  Record

your hypothesis below.  (Make sure it is an if…then statement!)

_____________________________________________________________________

_____________________________________________________________________

Insert the funnel into the can's opening.  Add some sand so that it is about half-filled.

Repeat the experiment, exactly as described above with the heavier can.  Record your

answers below.

First Peak:___________________seconds

Second Peak:___________________seconds

Second Peak:

-     First Peak:

Period:                        seconds

Are the periods significantly different for the light can and the heavy can?

_____________________________________________________________________

Does this result agree with your hypothesis?

_____________________________________________________________________

5.  How do you think the period would be affected if a shorter piece of string were used?

Record your hypothesis below.  (Make sure it is an if…then statement!)

_____________________________________________________________________

_____________________________________________________________________

Measure 25 centimeters along the string that is already attached to the can and cut the

string at this point.  Repeat the experiment, exactly as described with the shorter

string.  Record your results below.

First Peak:___________________seconds

Second Peak:___________________seconds

Second Peak:

-     First Peak:

Period:                        seconds

Are the periods significantly different for the longer string and the shorter string?

_____________________________________________________________________

Does this result agree with your hypothesis?

_____________________________________________________________________

6.  Complete the following statements based upon your observations during this activity.

(Use the words increase, decrease, or stay the same.)  Changing the weight of the

pendulum's hanging object causes its period to _____________________________,

and decreasing its length causes its period to _______________________________.

Going Further

Directions:  Answer these questions and show all work.  If you run out of room please

continue on a separate sheet of paper.

1.  In the second part of this activity, the pendulum's length was divided in half, and

this caused its period to change.  Was the period divided in half also?

__________________________________________________________________

__________________________________________________________________

What does this tell you about the relationship between the length and the period?

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

2.  The frequency of a swinging pendulum tells how many cycles it completes per

second.  Frequency and period are related by this simple formula:

Frequency =      1

Divided by

Period

Based on the data you collected, find the frequencies for the long-string pendulum

and the short-string pendulum.  Show all work and write your answers in a complete

sentence.

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Long-string  					Short-string

Pendulum  					Pendulum

3.  How would frequency be affected (increase, decrease, or remain the same) if the

pendulum's length increased?

_____________________________________________________________________

_____________________________________________________________________

4.  How would frequency change if a pendulum's period is tripled.  Be specific and use

numbers in an example to support your hypotheses.

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________